EXPERIMENT:	Flue Gas Analysis

OBJECTIVES:

- To identify the composition of flue gas resulted by a combustion process

- To determine the air fuel ratio using the composition of flue gas and compare with the theoretical air supply

- To make necessary action to have correct air fuel ratio

THEORY:

Theoretical air supply per 1 kg of fuel (r) 	=  1002383C+8H2+ S- O2 kg

C – Mass of carbon in 1 kg of fuel

H – Mass of hydrogen in 1 kg of fuel

S – Mass of sulfur in 1 kg of fuel

O – Mass of oxygen in 1 kg of fuel

Mass of flue gas per 1 kg of fuel (x) 		= carbon content in 1 kg of fuelcarbon content in 1 kg of flue gas

PROCEDURE:

- Start the combustion process and allow the process to become steady state

- Find the composition of flue gas using the gas analyzer

- Calculate the excess air factor

- Make necessary action to have correct excess air factor

CALCULATIONS:

- Calculate theoretical air supply per 1 kg of fuel

- Calculate mass of flue gas per 1 kg of fuel

- Calculate actual air supply per 1 kg of fuel

- Calculate excess air factor

RESULTS:

Indicate above results in a table.

DISCUSSION:

- What is the importance to have optimum excess air factor?

- What is the variation of optimum excess air factor with the states of fuels?

- Discuss practical applications

OBSERVATIONS

EXPERIMENT:	Flue Gas Analysis

EXPERIMENT NO:

INDEX NO:

| Components of Flue Gas | Composition |
| --- | --- |