EXPERIMENT:	Rankine Cycle

OBJECTIVES:

- To determine the Rankine cycle efficiency

APPARATUS:

- Steam motor set

- Stopwatch

- Measuring cylinder

TEORY:

The Rankine cycle is an ideal cycle for comparing the performance of steam plants. It is modified form of Carnot cycle, in which the condensation process (3-4) is continued until the steam is condensed into water. The schematic diagram of a steam engine is shown in figure.

Rankine cycle efficiency = ηR

ηR= hB-hFhB-hD

Heat supplied to the boiler;

qin= h2-h1mo				Where mo is the steam flow rate

h2= h1+ qinmo 				Where h1 = hf at boiler pressure

Dryness fraction of steam at state 2 is given by

h2= hA+ x2hB-hA

x2= h2-hAhB-hA

Dryness fraction of steam at state 3;

s2= sA+ x2sB-sA

But s2= s3

Also s3= sC+ x3sc- sD

x3= s3- sCsD- sC

Enthalpy of steam at state 3;

h3= hD+ x3hC- hD 			(hD = hf at atmospheric pressure)

PROCEDURE:

- Fill the boiler before switching on the heaters.

- Lower the spring balance to remove the load from the engine brake.

- When the boiler pressure becomes 60 kN/m2 (0.6 bar), open the stop valve until the engine inlet pressure reaches 40 kN/m2 (0.4 bar), to 50 kN/m2 (0.5 bar).

- Turn the steam motor wheel clockwise.

- Adjust the condenser-cooling water supply to a flow rate of about 100 l/hr.

- Adjust the engine speed to 2000 rpm. Allow the engine to run for at least 10 minutes before taking readings.

- Take necessary readings for different load applied by the spring balance.

- Note down the atmospheric temperature and pressure.

CALCULATIONS:

- Enthalpy of steam leaving the boiler.

- Enthalpy of steam leaving the steam motor.

- Enthalpy of water entering the pump

- Enthalpy of water entering the boiler

- Rankine efficiency of the steam plant.

DISCUSSION:

- Explain the difference between Carnot cycle & Rankine cycle

- Discuss practical errors.

OBSERVATIONS:

EXPERIMENT: 	DRYNESS FRACTION OF STEAM

EXPERIMENT NO:

ADMISSION NO:

|  | Test 01 | Test 02 |
| --- | --- | --- |
| Boiler pressure (kN/m2) |  |  |
| Watt meter reading |  |  |
| Time taken to collect 100 ml of condensates. |  |  |

Atmospheric Pressure (bar):	….......................................